Typical Bender operation – Normal mode
Creating multiple pieces of a shape with N lines,

- Shear and start [Full Re-home procedure]

  - Reset head angle – [Return the head to default CW home position, either for CCW or CW bending]

    - Determine if the next angle is cw or ccw bend

      - CW bends [Pin above rebar] – HEAD_CCW to trip proximity_head_cw

      - CCW bends [Pin below rebar] - HEAD_CW to trip proximity_head_ccw

    - How can we detect ‘limbo’? [What if head is past sensors on either side?]

      - PROXIMITY_HEAD_LIMIT sensor is likely at the edge of both halves of the head

      - If PROXIMITY_HEAD_LIMIT is tripped, reverse rotation until you hit the proper proximity sensor

      - Ex: For CW bends, if PROXIMITY_HEAD_LIMIT is tripped and HEAD_CCW is high, then change to HEAD_CW until proximity_head_cw is tripped

  - Retract head & forming tool

    - HEAD_IN to trip PROXIMITY_HEAD_IN

    - TOOL_IN to trip PROXIMITY_TOOL_IN

  - Home shear and cut once to reset material

    - SHEAR_HOME until PROXIMITY_SHEAR_HOME trips

    - SHEAR_CUT until PROXIMITY_SHEAR_CUT trips

    - SHEAR_HOME until PROXIMITY_SHEAR_HOME trips

  - Extend head/forming tool until interrupt

    - TOOL_OUT to trip PROXIMITY_TOOL_OUT

    - HEAD_OUT to trip PROXIMITY_HEAD_OUT

  - Feed ‘X’ length. [Feed the material so that it reaches a pre-defined length, at the end of the forming tool]

    - FEED_FORWARD until encoder_feed_set/_reset reaches ‘X’

      - Original machine uses both _set/_reset, ‘OR’ together. _reset is likely incremented by reverse movement

- Lines x N

[Assuming continuous operation. Any pauses require a full re-home, repeat step 1 above.]

  - Angle

    - Check if next line cw or ccw, and set head angle to proper position1

      - CW bends [Pin above rebar] – if proximity_head_ccw is tripped, retract head, head_ccw to trip proximity head cw, and extend head

      - CCW bends [Pin below rebar] – If proximity_head_ccw is tripped, retract head, HEAD_CW to trip proximity_head_ccw, and extend head

  - Feed

    - While materialFed < length*, feed

    - Every feed counter interrupt goes to previous step, increment on every interrupt

    - When materialFed >= length, move to bend

  - Bend

    - While materialBent < bend**, bend

    - Every bend counter interrupt goes to previous step, increment on every interrupt

    - When materialBent >= bend, unbend to reset

      - Count on interrupt to tell us the starting position, or do we need a counter?

  - Repeat step 2 N times, once for each line

- Finish the shape

  - Retract head/tool

    - HEAD_IN to trip PROXIMITY_HEAD_IN

    - TOOL_IN to trip PROXIMITY_TOOL_IN

  - Retract rebar ‘X’ length, standardized to bring correct length from shear

    - Should be roughly the same ‘x’ as in step 1.e

  - Cut shape

    - SHEAR_HOME until PROXIMITY_SHEAR_HOME trips [Limbo check]

    - SHEAR_CUT until PROXIMITY_SHEAR_CUT trips

    - SHEAR_HOME until PROXIMITY_SHEAR_HOME trips

  - Extend head/forming tool until interrupt

    - TOOL_OUT to trip PROXIMITY_TOOL_OUT

    - HEAD_OUT to trip PROXIMITY_HEAD_OUT

  - Increment runCount, return to Step 2 if runCount <= totalPieces

# Notes

- Re-start

  - If the machine has been stopped in the middle of a run, a full re-home is required

- 2 types of re-home

  - A full re-home is required at the start of a run, or resuming after a pause

    - Full ‘limbo’ checking

  - A mid-run re-home is required after every continuous shape is cut in a run

    - Because the machine is already running automatically, we can assume that the shear is home and that the head/tool in/out are home. Head angle will have to be checked, because each angle can be cw or ccw

1 – We can assume the head angle is at one of the sensor positions because the machine would be continuously running. Any pause requires a re-home operation.

*Length formula: (length + lengthCompensation) * feedScale * barSizeModifier
**Bend formula: (bend + bendCompensation) * centerPinModifier (?) barBendModifier

length = length programmed per line
lengthCompensation = compensation programmed per line
feedScale = systemwide modifier, programmed from settings
barSizeModifier = system adjustment for different sizes of rebar (4/8” vs 5/8”, etc)
bend = bend programmed per line

bendCompensation = bend compensation programmed per line

centerPinModifier = bend compensation for different center pin sizes

barBendModifier = bend compensation for different sizes of rebar

HMI logic - 
mid-run pause leads to two options ‘remake shape’ and ‘continue’. Remake shape does a full re-home, cutting any half-finished material immediately. Continue keeps going from where the run left off, at operator’s risk.